# Software Specification – Imaging Hardware

# Purpose

The Imaging Hardware software controls the image acquisition process on each Raspberry Pi imaging module. Each unit interfaces with a camera to capture high-resolution images of the drill core and transfer them to the control hardware for transfer to the PC.

# Hardware Platform

- Raspberry Pi ? Model ?

- Raspberry Pi Camera Module

- Local storage (SD Card)

- USB connection 2.0 Serial

# Software Environment

| Item | Specification |
| --- | --- |
| Operating System | Raspberry Pi OS Lite |
| Programming Language | Python 3 |
| Camera Library | Picamera2 |
| Network Communication | pySerial |
| File Handling | Python OS, Pathlib |

# Functional Requirements

## Image Capture

The software shall

- Initialise the camera

- Verify camera connectivity

- Capture images on command

- Save images in JPEG format

- Store images using unique filenames

Example filename: Camera03_Image00125.jpg

## Image Storage

The software shall:

- Store images locally

- Organise images into project folders

- Verify successful file creation

- Report storage failures

## Communication Interface

The software shall:

- Register with the Control Raspberry Pi during startup.

- Receive image capture commands over the USB serial bus.

- Capture an image when commanded.

- Store the image locally.

- Return completion status to the controller.

- Report hardware faults.

### Serial Commands

| Command | Description |
| --- | --- |
| ID | Return camera ID |
| STATUS | Return camera status |
| CAPTURE | Capture image |
| RESET | Restart software |
| SHUTDOWN | Safe shutdown |

## Health Monitoring

The software shall report when requested:

- Camera status

- Storage remaining

- Network connection

## Inputs

| Source | Description |
| --- | --- |
| Control Hardware | Capture command |
| Control Hadware | Status request |
| Control Hardware | File transfer |

## Outputs

| Destination | Description |
| --- | --- |
| Control Hardware | Capture complete message |
| Control Hardware | Eror messages |
| Control Hardware | Image Files |
| Local Storage | Image Files |

# Error handling

The software shall detect:

- Cammera disconnected

- Storage full

- Capture timeout

Recovery actions include:

- Retry Image Capture

- Log errors

# Logging

Record:

- Capture time

- Camera ID

- Image Filename

- Errors